Contents

2.	Introduction:	2

2.1	Problem Description:	2

2.2	Project Scope:	2

3.	Features of the Circuit:	2

3.1	Electrical Characteristics:	2

3.2	Components Used:	3

3.3	Distinguishing Features of Experiment:	3

4.	Flow of Experiment:	3

4.1	Circuit Configuration:	3

4.2	Circuit Diagram:	4

4.3	Physical Circuit:	4

5.	Technical Concepts:	5

5.1	RLC Series Theory:	5

5.2	Energy Storage Equations:	5

6.	Challenges Faced During Experiment:	5

7.	Calculations:	6

7.1	Resonant (natural) angular frequency ω0:	6

7.2	Damping factor α:	6

7.3	Characteristic roots s:	6

7.4	Time constants (magnitudes):	6

7.5	50% (t₅₀) or “time-delay” estimate:	7

8.	Observations:	7

9.	Conclusion:	7

# Introduction:

## Problem Description:

- The objective of this experiment is to analyze an Acceptor (RLC Series) resonant circuit and study its transient energy storage characteristics. A 9V supply energizes the circuit, and the LED glow duration after removing the supply reflects the energy stored in the inductor and capacitor.

The system demonstrates:

- Energy stored in capacitor & inductor

- Slow discharge through resistance

- Overdamped transient behavior

## Project Scope:

This experiment focuses on:

- Understanding RLC behavior without AC resonance testing

- Measuring stored energy visibility through LED light

- Observing the extended discharge response

# Features of the Circuit:

## Electrical Characteristics:

- Series-connected components

- High resistance that causes slow decay

- LED to demonstrate energy storage

- Capacitor stores electric energy, and inductor stores magnetic energy

## Components Used:

| Component | Value |
| --- | --- |
| Resistor 1 | 15 kΩ |
| Resistor 2 | 4.7 kΩ |
| Inductor (L) | 20 mH |
| Capacitor (C) | 2200 µF |
| LED | Standard Blue LED |
| Power Source | 9V DC |

## Distinguishing Features of Experiment:

- Long LED after-glow (approximately 4.5 minutes)

- High-resistance overdamped response

- Slow exponential energy decay visually measurable

- Non-AC RLC demonstration (DC charging)

# Flow of Experiment:

## Circuit Configuration:

- 9V Supply

- 15 kΩ Resistor

- 20 mH Inductor

- 2200 µF Capacitor

- 4.7 kΩ Resistor

- LED

## Circuit Diagram:

  - Multisim:

  - Graph:

## Physical Circuit:

# Technical Concepts:

## RLC Series Theory:

Current remains same in the loop but:

- Inductor opposes sudden change in current

- Capacitor opposes sudden change in voltage

## Energy Storage Equations:

  - Capacitor:

EC=12CV2

  - Inductor:
EL=12LI2

# Challenges Faced During Experiment:

| Area | Challenge |
| --- | --- |
| Component Selection | Ensuring safety for LED |
| Measurement | LED’s faint glow hard to detect in room light |
| Time Recording | Manual stopwatch introduces error |

# Calculations:

R=19.7 KΩ=19.7 ×103 Ω  = 19, 700 Ω

C=2200 μF=2200×10-6F=0.0022 F

L=10mH=10×10-3H=0.01 H

## Resonant (natural) angular frequency ω0:

ω0= 1LC

ω0= 10.01*(0.0022)

ω0 = 213.2 rad/sec

## Damping factor α:

α= R2L

α= 19,7002(0.01)

α= 985,000 Np/sec

Since α is greater than ωo, The damping will be overdamped.

## Characteristic roots s:

s = - α ± α2-ωo2

s= -985,000 ± (985,000)2-(213.2)2

s= -985,000 ± 984,999.9769

s1= -985,000+ 984,999.9769 s2= -985,000- 984,999.9769

s1= -0.0231 s-1                                       s2 = -1,969,999.977 s-1

## Time constants (magnitudes):

T1= -1s1=  -1-0.0231=43.34 sec

T2= -1s2= -1-1,969,999.977=5.08 ×10-7 sec

The dominant time constant is τ1≈43.34 s

## 50% (t₅₀) or “time-delay” estimate:

For an exponential decay, the time to reach 50% is about

t50​≈0.693τ1​≈0.693×43.34≈30.0 seconds.

# Observations:

- Brightness ≈ 20–25 sec

- Faint visibility ≈ 4.5 minutes

- Smooth decay – no flicker

# Conclusion:

The experiment confirms that an RLC series circuit can store and gradually release energy, demonstrating an overdamped transient response. The LED glow proves prolonged discharge behavior when resistance is high.